NETWORK ENGINEERING LAB REPORT

Multi-Area OSPF

EVE-NG Community Edition  |  Cisco c7200 (Dynamips)  |  Proxmox VE

| Author | Sujan Thapa |
| --- | --- |
| Date | 2026-04-21 |
| Lab Platform | EVE-NG Community on Proxmox VE |
| Router Image | Cisco c7200 (Dynamips) |
| OSPF Process | Process ID 1 3 Areas (0, 1, 2) |
| Status | Completed |

# 1. Objective

This lab demonstrates the design, configuration, and verification of a multi-area OSPF network across three areas (Area 0, Area 1, Area 2) using six Cisco c7200 routers in EVE-NG. The lab covers ABR configuration, inter-area routing, DR/BDR behaviour, point-to-point network types, and real-world troubleshooting of adjacency issues.

Skills demonstrated are directly aligned with enterprise NOC roles requiring OSPF expertise in production environments.

## 1.1 Learning Objectives

- Configure multi-area OSPF with correct area assignments across 6 routers

- Understand and configure Area Border Routers (ABR) between areas

- Resolve real-world adjacency issues including duplex mismatch and network type mismatch

- Verify inter-area routing using O IA routes in the routing table

- Understand DR/BDR election and eliminate it using point-to-point network type

- Document troubleshooting methodology for a professional portfolio

# 2. Lab Topology

## 2.1 Topology Diagram

The topology consists of 6 routers spanning 3 OSPF areas. R2 and R3 are ABRs connecting Area 1 to the backbone (Area 0). R4 and R5 are ABRs connecting Area 0 to Area 2. R4 is the most connected router with 4 interfaces spanning both Area 0 and Area 2.

[ INSERT EVE-NG TOPOLOGY SCREENSHOT HERE ]

Figure 1: Multi-Area OSPF Topology — EVE-NG Canvas

## 2.2 Area Design

| Area 1 | Non-backbone area — contains R1 (internal), R2 (ABR), R3 (ABR) |
| --- | --- |
| Area 0 | Backbone area — contains R4 (ABR), plus Area 0 interfaces of R2, R3, R5 |
| Area 2 | Non-backbone area — contains R5 (ABR), R4 (ABR), R6 (internal) |
| ABR Count | 4 ABRs total — R2, R3, R4, R5 |
| Internal Routers | R1 (Area 1 only), R6 (Area 2 only) |
| Redundancy | Dual ABRs per area boundary — provides path redundancy |

## 2.3 ABR Role Summary

| Router | Areas | Role | Description |
| --- | --- | --- | --- |
| R1 | Area 1 only | Internal | Pure Area 1 router — no backbone connection |
| R2 | Area 1 + Area 0 | ABR | Connects Area 1 to backbone via e0/1 |
| R3 | Area 1 + Area 0 | ABR | Redundant ABR for Area 1 alongside R2 |
| R4 | Area 0 + Area 2 | ABR | Central hub — 4 interfaces, connects both sides |
| R5 | Area 0 + Area 2 | ABR | Redundant ABR for Area 2 alongside R4 |
| R6 | Area 2 only | Internal | Pure Area 2 router — dual connection to R4 and R5 |

# 3. IP Addressing Plan

## 3.1 Loopback Interfaces (Router-IDs)

| R1 Loopback0 | 1.1.1.1/32 — Area 1 |
| --- | --- |
| R2 Loopback0 | 2.2.2.2/32 — Area 1 |
| R3 Loopback0 | 3.3.3.3/32 — Area 1 |
| R4 Loopback0 | 4.4.4.4/32 — Area 0 |
| R5 Loopback0 | 5.5.5.5/32 — Area 2 |
| R6 Loopback0 | 6.6.6.6/32 — Area 2 |

## 3.2 Full Interface IP Table

| Router | Interface | IP Address | Connected To | Area | Role |
| --- | --- | --- | --- | --- | --- |
| R1 | fa0/0 | 10.0.12.1/30 | R2 e0/0 | 1 | Internal |
| R1 | fa0/1 | 10.0.13.1/30 | R3 fa0/0 | 1 | Internal |
| R2 | e0/0 | 10.0.12.2/30 | R1 fa0/0 | 1 | ABR |
| R2 | e0/1 | 10.0.24.1/30 | R4 fa0/0 | 0 | ABR |
| R3 | fa0/0 | 10.0.13.2/30 | R1 fa0/1 | 1 | ABR |
| R3 | fa0/1 | 10.0.34.1/30 | R4 fa1/0 | 0 | ABR |
| R4 | fa0/0 | 10.0.24.2/30 | R2 e0/1 | 0 | ABR |
| R4 | fa1/0 | 10.0.34.2/30 | R3 fa0/1 | 0 | ABR |
| R4 | fa2/0 | 10.0.45.1/30 | R5 fa0/0 | 0 | ABR |
| R4 | fa3/0 | 10.0.46.1/30 | R6 fa0/0 | 2 | ABR |
| R5 | fa0/0 | 10.0.45.2/30 | R4 fa2/0 | 0 | ABR |
| R5 | fa0/1 | 10.0.47.1/30 | R6 fa0/1 | 2 | ABR |
| R6 | fa0/0 | 10.0.46.2/30 | R4 fa3/0 | 2 | Internal |
| R6 | fa0/1 | 10.0.47.2/30 | R5 fa0/1 | 2 | Internal |

# 4. OSPF Configuration

## 4.1 R1 — Internal Area 1 Router

| R1 OSPF | router ospf 1 router-id 1.1.1.1 log-adjacency-changes network 1.1.1.1 0.0.0.0 area 1 network 10.0.12.0 0.0.0.3 area 1 network 10.0.13.0 0.0.0.3 area 1 |
| --- | --- |

## 4.2 R2 — ABR (Area 1 + Area 0)

| R2 OSPF | router ospf 1 router-id 2.2.2.2 network 2.2.2.2 0.0.0.0 area 1 network 10.0.12.0 0.0.0.3 area 1 network 10.0.24.0 0.0.0.3 area 0 |
| --- | --- |

## 4.3 R3 — ABR (Area 1 + Area 0)

| R3 OSPF | router ospf 1 router-id 3.3.3.3 network 3.3.3.3 0.0.0.0 area 1 network 10.0.13.0 0.0.0.3 area 1 network 10.0.34.0 0.0.0.3 area 0 |
| --- | --- |

## 4.4 R4 — ABR (Area 0 + Area 2)

| R4 OSPF | router ospf 1 router-id 4.4.4.4 network 4.4.4.4 0.0.0.0 area 0 network 10.0.24.0 0.0.0.3 area 0 network 10.0.34.0 0.0.0.3 area 0 network 10.0.45.0 0.0.0.3 area 0 network 10.0.46.0 0.0.0.3 area 2 |
| --- | --- |

## 4.5 R5 — ABR (Area 0 + Area 2)

| R5 OSPF | router ospf 1 router-id 5.5.5.5 log-adjacency-changes network 5.5.5.5 0.0.0.0 area 2 network 10.0.45.0 0.0.0.3 area 0 network 10.0.47.0 0.0.0.3 area 2 |
| --- | --- |

## 4.6 R6 — Internal Area 2 Router

| R6 OSPF | router ospf 1 router-id 6.6.6.6 log-adjacency-changes network 6.6.6.6 0.0.0.0 area 2 network 10.0.46.0 0.0.0.3 area 2 network 10.0.47.0 0.0.0.3 area 2 |
| --- | --- |

## 4.7 Point-to-Point Network Type

All interfaces were configured with point-to-point network type to eliminate unnecessary DR/BDR election on /30 links:

| All Routers | interface <interface-name> ip ospf network point-to-point |
| --- | --- |

This eliminates Type 2 Network LSAs, speeds up adjacency formation, and removes DR/BDR election overhead on point-to-point /30 segments.

# 5. Issues Encountered & Resolutions

The following issues were encountered during lab configuration and resolved through systematic troubleshooting. Each issue demonstrates real-world diagnostic skills applicable to a production NOC environment.

| Issue | Root Cause | Resolution |
| --- | --- | --- |
| R1 loopback not reachable from any router | network 1.1.1.1 0.0.0.0 area 1 missing from R1 OSPF config — loopback not advertised | Added network statement for 1.1.1.1/32 under router ospf 1 on R1 |
| R1 ↔ R3 adjacency never formed | Duplex mismatch — R3 fa0/0 was full duplex, R1 fa0/1 was half duplex. CDP warning visible in show ip ospf output | Set duplex full and speed 100 on both R1 fa0/1 and R3 fa0/0 |
| Network type mismatch on R1 ↔ R3 link | R3 fa0/0 was BROADCAST, R1 fa0/1 was POINT_TO_POINT — OSPF requires matching network type on both ends | Set ip ospf network point-to-point on R3 fa0/0 |
| Wrong subnet mask on R1 fa0/1 | Interface configured with /24 instead of /30 — OSPF network statement not matching correctly | Corrected to ip address 10.0.13.1 255.255.255.252 |
| R1 routing table empty despite adjacency with R2 | Network type mismatch causing SPF calculation failure — Type 2 Network LSA present when it should not be | Fixed network type to point-to-point on all interfaces |
| DR/BDR election on all links | FastEthernet/Ethernet interfaces default to broadcast network type — triggers unnecessary DR/BDR election on /30 links | Applied ip ospf network point-to-point on all interfaces across all 6 routers |
| High OSPF cost on R1-R2 link (cost 11) | Mixed interface types — Ethernet (10Mbps reference) vs FastEthernet (100Mbps reference) giving different auto-cost calculations | Set auto-cost reference-bandwidth 1000 on all routers to normalise costs |

# 6. Verification

## 6.1 Key Verification Commands

| show ip ospf neighbor | Verify all neighbors in FULL state with no DR/BDR (shows FULL/ -) |
| --- | --- |
| show ip ospf | Confirm router is ABR — shows 'It is an area border router' |
| show ip route ospf | Verify O (intra-area) and O IA (inter-area) routes present |
| show ip ospf database | Confirm LSA Types 1, 2, 3 present — Type 3 are inter-area summaries from ABRs |
| show ip ospf interface | Confirm network type POINT_TO_POINT and correct area assignment |
| ping <loopback-ip> | End-to-end reachability test across all 3 areas |

## 6.2 Expected Neighbor States

After full configuration, all neighbors should show FULL/  - (point-to-point, no DR/BDR):

| R4# show ip ospf neighbor | Neighbor ID Pri State Dead Time Address Interface 5.5.5.5 0 FULL/ - 00:00:37 10.0.45.2 FastEthernet2/0 3.3.3.3 0 FULL/ - 00:00:31 10.0.34.1 FastEthernet1/0 2.2.2.2 0 FULL/ - 00:00:33 10.0.24.1 FastEthernet0/0 6.6.6.6 0 FULL/ - 00:00:33 10.0.46.2 FastEthernet3/0 |
| --- | --- |

## 6.3 Expected Routing Table on R1

R1 should see O IA (inter-area) routes to all remote loopbacks and subnets:

| R1# show ip route ospf | O 2.2.2.2/32 [110/2] via 10.0.12.2, FastEthernet0/0 O 3.3.3.3/32 [110/2] via 10.0.13.1, FastEthernet0/1 O IA 4.4.4.4/32 [110/3] via 10.0.12.2, FastEthernet0/0 O IA 5.5.5.5/32 [110/4] via 10.0.12.2, FastEthernet0/0 O IA 6.6.6.6/32 [110/4] via 10.0.12.2, FastEthernet0/0 O IA 10.0.45.0/30 [110/3] via 10.0.12.2, FastEthernet0/0 O IA 10.0.46.0/30 [110/3] via 10.0.12.2, FastEthernet0/0 O IA 10.0.47.0/30 [110/4] via 10.0.12.2, FastEthernet0/0 |
| --- | --- |

## 6.4 End-to-End Ping Test

Verified reachability between all loopbacks across all three areas:

| Source | Destination | Path | Result |
| --- | --- | --- | --- |
| R1 (1.1.1.1) | R6 (6.6.6.6) | Area 1 → 0 → 2 | Passes through R2/R3 ABR → R4 → R6 |
| R3 (3.3.3.3) | R6 (6.6.6.6) | Area 1 → 0 → 2 | Direct via R3 ABR → R4 → R6 |
| R1 (1.1.1.1) | R4 (4.4.4.4) | Area 1 → 0 | Via R2 or R3 ABR |
| R6 (6.6.6.6) | R1 (1.1.1.1) | Area 2 → 0 → 1 | Reverse path via R4/R5 → R2/R3 |

7. Key Concepts Demonstrated

## 7.1 DR vs BDR vs ABR

|  | DR | BDR | ABR |
| --- | --- | --- | --- |
| Purpose | Reduces adjacencies on broadcast segments | Backs up the DR — takes over if DR fails | Connects two OSPF areas together |
| Where | Broadcast networks (Ethernet) | Broadcast networks (Ethernet) | Any router with interfaces in 2+ areas |
| Elected | Yes — by priority then router-id | Yes — second highest wins | No — automatic based on interface areas |
| In this lab | Eliminated by point-to-point | Eliminated by point-to-point | R2, R3, R4, R5 |
| LSA Type | Generates Type 2 Network LSA | None | Generates Type 3 Summary LSA |

## 7.2 OSPF Network Types

| Broadcast (default) | DR/BDR election occurs. Used on multi-access Ethernet segments. Generates Type 2 LSA. |
| --- | --- |
| Point-to-Point | No DR/BDR. Used on /30 links between exactly 2 routers. No Type 2 LSA generated. |
| Why P2P on /30 links | On a /30 only 2 hosts exist — DR/BDR election is wasteful. P2P is faster and cleaner. |
| Command used | ip ospf network point-to-point on each interface |

## 7.3 OSPF network Command

The network command does NOT automatically discover interfaces. It matches interfaces whose IP falls within the specified range and area assignment:

| Syntax | network <network-address> <wildcard-mask> area <area-id> ! Examples: network 1.1.1.1 0.0.0.0 area 1 ! exact match - loopback only network 10.0.12.0 0.0.0.3 area 1 ! matches 10.0.12.1 and 10.0.12.2 network 0.0.0.0 255.255.255.255 area 0 ! matches ALL interfaces (not recommended) |
| --- | --- |

# 8. Lessons Learned

- Always set router-id explicitly before starting OSPF — avoids unexpected ID changes if interfaces come up in different order

- Set ip ospf network point-to-point on all /30 links — eliminates DR/BDR overhead and simplifies troubleshooting

- Loopback interfaces must be explicitly included in the network statement — OSPF does not auto-discover them

- Duplex mismatch silently breaks OSPF adjacency — always check show interface for Full-duplex confirmation

- Network type must match on both ends of a link — BROADCAST vs POINT_TO_POINT mismatch prevents adjacency

- Having dual ABRs per area boundary (R2+R3 for Area 1, R4+R5 for Area 2) provides redundancy — if one ABR fails the other maintains connectivity

- CDP warning messages in show ip ospf output are valuable diagnostic clues — always read full command output

- The show ip ospf database command reveals what LSAs exist even when the routing table is empty — useful for diagnosing SPF failures

# 9. Next Steps

| Project 2 | Add loopbacks to R1 and R6, configure area range summarization on ABRs, verify Type 3 LSA reduction |
| --- | --- |
| Project 3 | Configure OSPF redistribution — add static routes on R1 as ASBR, understand Type 5 LSAs |
| Project 4 | Configure MD5 authentication on all OSPF interfaces, simulate auth failure and recovery |
| BGP Lab | Add eBGP peering between a new router and R1 — simulate ISP connectivity as Bloomberg does with exchanges |
| Python Automation | Write Netmiko script to pull show ip ospf neighbor from all routers and alert on non-FULL state |
| Grafana Monitoring | Configure SNMP on all routers, build Grafana dashboard showing OSPF neighbor state and link utilisation |

# Appendix — Quick Reference Commands

| show ip ospf neighbor | Check all adjacencies and their state |
| --- | --- |
| show ip ospf | Confirm ABR role and area count |
| show ip ospf interface | Verify network type, area, cost, DR/BDR state |
| show ip ospf database | View all LSAs in the LSDB |
| show ip ospf database router | View Type 1 Router LSAs |
| show ip ospf database network | View Type 2 Network LSAs |
| show ip ospf database summary | View Type 3 Summary LSAs from ABRs |
| show ip route ospf | View OSPF routes — O = intra-area, O IA = inter-area |
| show interface <intf> | Check duplex, speed, input/output errors |
| debug ip ospf adj | Watch adjacency formation in real time |
| clear ip ospf process | Reset OSPF — use after router-id change |
| ping <ip> source <lo0> | Ping using loopback as source — tests OSPF reachability |